eNORA2 user manual

About the setting of user variables

There are a number of user variables to be set before eNORA2 is initiated. The user variables are labeled with user.variableName. Strings are given with single quotation marks, such as user.projectname = 'PROJECTNAME' and multiple numbers are given in vector form such as user.mixing_times = [0.02 0.03 0.04 0.05]. All input data such as pdb structures, eNOE and rho data is to be placed into the input_files directory.

All output is found in the output_files/PROJECTNAME directory. There is the possibility to save the current status of the project and to continue the analysis later  (see  below ‘On the fly’ section for more details).

A standard setup script to run eNORA2 is provided with the eNORA2 routine (Figure 1.).

Running eNORA2

The setup script

Fig. 1. Standard setup file. Double commented (%%) commands are optional commands used as described in the text.

Essential user variables

Projectname:

user.projectname

The project name is typically set to a string identifying the data used. It is the name the project will be given trailing the time stamp, i.e. user.projectname = 'PROJECTNAME' will be applied to create a directory such as /output_files/160322_15:33_PROJECTNAME.

Structure for spin diffusion calculation:

user.pdbFileName, user.pdbNameConvention

The spin diffusion calculation requires a structure coordinate file. If your starting structure is a crystal structure make sure you provide proton atom positions. Provide a pdb file with just one model and with protons and pseudo atoms. Usually the lowest energy model is used. In order to read the structure file the naming convention of the protons has to be given. The three possible options are markley.lib [Ref. 1], cyana.lib and oldcyana.lib. Once you have edited your pdb structure file to contain pseudo atoms and protons set user.pdbFileName = 'pdbName.pdb' and user.pdbNameConvention = 'namingConvention.lib'.

Computer programs such as CYANA can add the protons and the pseudo atoms to structures derived by X-ray crystallography.

Reading the buildup data:

user.specDimens, user.nmrDrawTabFileName, user.intensityFileName, user.assNameConvention

To read buildup data, the user is required to specify the dimensionality of the NOESY experiment used, by setting user.specDimens = 2 or 3.

For buildup input files, there are two options. Either use an NMRDRAW file or a generic intensity file described below. Peak assignment of an NMRDRAW.tab file needs to conform to the following assignment standard:

res.proton;res.proton;res.heavyatom, i.e. 16.H;22.HA;22.CA (stored in sigmaData.input.ASS) for a three dimensional experiment and 16.H;22.HA for a two dimensional NOESY experiment. Once an NMRDRAW.tab file containing the complete assignment has been generated, use of the fitting tool of nmrPipe (series3D.com for 3D NOESY and fit.com for 2D NOESY) adds the relative intensity variables A0, … , AN for N+1 mixing times), where A0 should be the reference spectrum, in which the resonances were assigned (typically at the longest mixing time). Remove the following to lines from the header (NULLVALUE -666 and NULLSTRING *) then your input file would look like Figure 2. Set user.nmrDrawTabFileName = 'NMRDRAWFILE.tab' and user.assNameConvention = ' namingConvention.lib' according to the assignment standard you used. The three possible options for naming conventions are markley.lib [Ref. 1], cyana.lib and oldcyana.lib, whereby the naming convention of the structure file, 'pdbName.pdb', and the buildup input file, 'NMRDRAWFILE.tab', are not required to be the same.

Fig. 2: Example of nmrDraw tab file entry format (user.nmrDrawTabFileName).

The second NOESY data input format allowed in eNORA2 is in the form of an intensity file and needs to follow the following assignment standard (Figure 3):

res aminoacid proton res aminoacid proton relativeIntensityList,

where the first entry of the relativeIntensityList is the assigned spectrum (typically at the longest mixing time).

i.e  13 LYS H 16 SER HA 1  0.8  0.6  0.4  0.2

To use this data input option, set user.intensityFileName = 'intensity_file' and user.assNameConvention = 'namingConvention.lib'. The three possible options for naming conventions are markley.lib [Ref. 1], cyana.lib and oldcyana.lib, whereby the naming convention of the structure file and the 'intensity_file' are not required to be the same.

Fig. 3: Example of intensity file entry format (user.intensityFileName). Columns 1 to 6 are the cross and diagonal peak assignments and 7 to10 the measured intensities at four different mixing times (in this specific case scaled to 1.0, which is not required).

Flow of the magnetization in the experiment, normalization:

user.magOriginSpin, user.magNormSpin

The assignment of the cross peaks in a NOESY experiment may be done such that the first entry indicates the origin of the magnetization and the destination of magnetization is the second entry (res.originSpin;res.destinationSpin) or vice versa (res.destinationSpin;res.originSpin), see Figure 4a. In a three dimensional experiment the last entry is then to be assigned to heavy atom such as res.originSpin;res.destinationSpin;res.heavyAtom or res.destinationSpin;res.originSpin;res.heavyAtom, see Figure 4b. If the first entry is the origin of magnetization set user.magOriginSpin = 1, otherwise set user.magOriginSpin = 2. The diagonal peak to be used for normalization may be the one of the origin or destination of the spin magnetization flow as chosen by the user (Figure 4).  Either set user.magNormSpin = 1 or 2.  This follows the logic of equations 8.1 and 8.2 or 8.3 and 8.4 in the accompanying manuscript.

Fig 4. Flow of magnetization in a typical a) 2D and b) 3D NOESY experiment.

Fitting of experimental data, user supplied information:

user.mixing_times, user.tauC , user.rhoAverage, user.perCentD2O, user.buildupRhoFileName

In order to fit the buildups the mixing times have to be given in units of seconds with the shortest mixing time given first (i.e. user.mixing_times = [0.02 0.03 0.04 0.05]).

Sample specific information requires the measured correlation time (i.e. user.tauC = 4.25) given in units of nanoseconds and the percent D2O added to the sample for deuterium locking  (i.e. user.perCentD2O = 3).

For an initial calculation of autorelaxation rates by eNORA2 from diagonal peaks set user.buildupRhoFileName = 'n'. Ideally, all autorelaxation rates are obtained from fitting the diagonal peak decays. These autorelaxation rates will be written into ‘RhoOut.txt’. However, in practice many diagonal peak decays cannot be fitted (due to spectral overlap, artifacts, etc.). Therefore, a general average autorelaxation rate for missing values may also be set by the user in units of s-1 (user.rhoAverage) in order to increase the number of eNOEs calculated. In practical cases, the influence of a value off by 1-2 s-1 is maximally 10% on the cross-relaxation rate [Ref. 2].  The missing, not fitted, autorelaxation rates are listed in the

output_files/final/‘NORhoOutNumCode.txt’ file.

Following the initial run, the estimation of the missing autorelaxation rates may be improved by using averages of the fitted experimental values (in the initial run) obtained for the same spin types. The procedure to calculate these average autorelaxation rates involves using the script calcAverageRhoConst.m found in the eNOEprogram/handTools directory or may be done manually by analyzing the values in the ‘RhoOut.txt’ file following the intial run of eNORA2 found in the buildup directory of the output_files/PROJECTNAME/buildup directory. The averages calculated per spin type are then manually written into the ‘NORhoOutNumCode.txt’ file, replacing the placeholder ‘rho’ with the respective value. The final input file (encompassing both the fitted experimental values and estimated values supplied for missing cases) is provided with the variable user.buildupRhoFileName. The file is a combination of the ‘RhoOut.txt’ and ‘NORhoOutNumCode.txt’ files, leaving out any headers of the respective files (Figure 5).

The numeric code used in the ‘NORhoOutNumCode.txt’ file to name the atom types can be understood by looking at the library file (user.assNameConvention) or comparing ‘NORhoOutNumCode.txt’ and ‘NORhoOutLeterCode.txt’.

Following the fitting of experimental buildups, the result is written into the output_files/final/ sigmaDataOutIso.txt  file (Figure 6).

Fig. 5: Example entries in a rho input file (user.buildupRhoFileName). The first column is the residue number, the second column the proton atom type given in numeric code, the third column is the autorelaxation rate in s-1, the fourth the Izero value calculated and the fifth column is the chi value calculated for the fit. The value 999 in the Izero column and chi column specify rho values calculated from experimental averages but not fitted from diagonal peak decay.

Fig. 6: Example entries in a sigmaDataOutIso.txt file.

Spin diffusion method:

user.spinDiffApp, user.B0field, user.stereoAvg, user.corrSigStereo

The spin diffusion method used by default is set to full matrix approach. For deuterated samples, a three spin approach is available [Ref. 3]. If you wish to use the three spin approach set user.spinDiffApp = 'threeSpin', otherwise the default 'fullMatrix' will be used.

In order to calculate the spin diffusion, the magnetic field strength (in MHz, ie. user.B0field = 700) is needed.

Spin diffusion via methyl groups is calculated by considering the three pathways given by the three individual protons.

If stereo specific information is not known for all spins the user should set the individual values of the calculated spin diffusion correction factor to an average of the stereo partners by setting user.stereoAvg = 1 and user.corrSigStereo = 'n'. If this information is known set user.stereoAvg = 0, or if it is partially known set user.stereoAvg = 1 and provide the known stereo and sequence specific atoms in numeric code in a text file (Figure 7) and setting the variable user.corrSigStereo = 'stereoSpecFileName'. This partially known stereospecific assignment information is then applied, for the non stereospecific NOEs an average spin diffusion corrections is applied.

Fig. 7. Example of a stereo center specification file (user.corrSigStereo). The first column is the residue number, the second and third columns are the known stereo centers in numeric code. Make sure to always provide both partners belonging to a stereo center.

Experimental restraints:

cyanaWrite.m

Running the eNORA2 master file generates the output file sigmaDataOutIso.txt in the output_files/TIMESTAMP_PROJECTNAME/final folder (Figure 8). This file contains the cross-relaxation rate constants  and  (SigmaExpCorrij and SigmaExpCorrji), and the effective internuclear distances  and  (rEffij and rEffji), their averages if both pathways are used (AvgSigma and AvgrEff), along with other parameters such as the factor by which that rate is scaled as compared to value obtained from a simple fit with the two-spin model, pij and pji (Pcorrij and Pcorrji), and the quality factors of the buildup fits, χ (Chi). Some figures are generated by default, the most important ones of which show the eNOE network superimposed on the input PDB structure, spin-diffusion correction factors and comparisons of the experimental cross-relaxation rates and distances to those predicted from the input structure (see Figure 7 in the manuscript).

Fig 8. Example entries in a sigmaDataOutIso.txt file.

Once the master file is run, the user has the option to create upper and lower limit distance restraint files in CYANA format (upl and lol files, see Figure 9). The cyanaWrite.m command generates these files by applying all the corrections listed in Table 1 in the manuscript.

Fig 9. Example of entries in a typical CYANA lol/upl file.

Matters of convenience:

user.autoPlot, user.assFilter, user.absIntensities

By default auto-plotting of all buildups and diagonal decays is disabled. One can enable this option by setting user.autoPlot = 0. Plots in png format will be saved into the buildupPlots and diagDecayPlots folders.

If the structure file does not match the assignment for some specific residues, an initial run of eNORA2 can be done by excluding NOEs from or to specific residues from calculation by setting (user.assFilter  = [resNumber, resNumber, resNumber …], the values indicating the mutated or missing residues).

Three dimensional NOESY experiments usually contain folded peaks, therefore the default option is set to use absolute valued intensities in order to avoid negative buildup plots (user.absIntensities = 1).

'On the fly'

loadPreviousResults 			this script is placed in the top directory of the program

and allows loading of earlier data sets

helpMenu				loads the help menu and displays the most common set of

commands a user may want to use 'on the fly'

search					finds eNOE entries in the sigma data structure

searchDiagDecay			finds entries in the diagDecay data structure

plotR					plots the calculated effective distance against the distance

determined from the input structure coordinates

plotS					plots the synthetic calculated sigma values against measured ones

molView				opens a structure viewer to view specific eNOEs

buildupViewer				plots specific buildups

plotAllBuildups				plots all buildups and saves them individually in the directory

output_files/PROJECTNAME/buildupPlots

plotAllDecays				plots all diagonal decays and saves them individually in the

directory output_files/PROJECTNAME/diagDecayPlots

cyanaWrite				writes out the CYANA upl and lol file interactively asking for

additional parameter settings; the user has to specify weather to

plot bi/uni-directional eNOEs or both and if fixed distances (structurally useless) constraints should be omitted;

the user is asked to provide a cutoff for the buildup fit quality (chi number) of the eNOEs to be used; furthermore,  the default tolerance value of the distance (20% for uni-directional eNOEs) can be changed

cyanaWriteGenNOE			writes out cyana upl and lol files if generic normalization was used

showNum				displays the number code (‘numcode’) for specific residues

saveData				saves the workspace to be reloaded later by the user

Optional user options for setup

Generic normalized eNOE:

user.IzeroMethyl, user.IzeroSingle, user.IzeroMethylenePseudo

If no generic limits for normalization are provided (see generic normalized eNOE) then non-normalized cross peaks are removed from the data to avoid nonsense buildup plots and nonsense points on other plots. In order to keep non-normalized peaks anyway add the user.noIz0Filter variable to your setup script and set it to 0. Then set the user.IzeroMethyl , user.IzeroSingle, user.IzeroMethylenePseudo (here, pseudo aromatics are also included in this group) to the generic value for each group in your setup script. The generic values are selected by calculating the upper limits of each group following the exclusion of specific risk groups (Ref. 4). Buildups normalized to these limits may then be used to generate upper limit distance restraints (Ref. 4).

Options for spin diffusion calculation:

user.spinDiffSim , user.spinDiffApp, user.labelingScheme, user.deut_HA, user.deut_Met, user.deut_Prot, user.upperlimit, user.rhoSupValues

If the user only wants to fit the buildups without correction for spin diffusion set user.spinDiffSim = 0.

For deuterated samples with a selective labeling scheme, set the labeling scheme, together with the percent deuteration level and the expected or measured deuteration levels and set the user.spinDiffApp to 'threeSpin'. In the basic setting, the deuteration levels of HN (given by the deuteration of D2O in the solution), HA, methylene and all other protons must be chosen separately with the variables user.perCentD2O, user.deut_HA, user.deut_Met and user.deut_Prot.

In addition, the user can also provide a methyl-group specific labeling scheme for methyl-bearing amino acids specified as a string and set with the variable user.labelingScheme = 'RES_MethGroup%0'. The available user input vocabulary covers all currently known selective labeling schemes and includes ALA_B, ILE_G2, ILE_D1, LEU_D1, LEU_D2, LEU_D1_D2, MET_E, THR_G2, VAL_G1, VAL_G2 and VAL_G1_G2. As an example, a protein selectively labeled as ILV would therefore be set as user.labelingScheme  = 'VAL_G1_G2%0 LEU_D1_D2%0 ILE_D1%0', if the deuteration level for the selectively labeled residues is 0%.

For spin diffusion calculation the upper limit for the distance of spins pairs to be included in the pathways is needed. The default limit is set to 6.5 Ångstroms, the limit can be changed by setting (user.upperlimit = X, in units of Ångstroms).

If a measurement point for a given mixing time is erroneous the user can remove that measurement point for all buildups and decays of diagonals simultaneously without editing the input file by setting (user.runThrough = 0, set to 1 to disable this option). This user setting temporarily interrupts the routine following an intial fitting of the buildup intensities and allowing the user to edit the time points. Accepting the changes will then refit the buildups with the chosen time points and run the routine until completion.

With the default setting of user.rhoSupValues = 1, the fitted autorelaxation rates are applied in the spin diffusion simulation in the three spin approach. If user.rhoSupValues = 0, simulated values will be used.

Non-standard amino acids and other molecules:

Non-standard amino residues or molecules other than proteins can be added to the numcode.lib file in the eNOEprogram/read_in folder in order to allow the program to recognize them. Simultaneously, an additional nomenclature convention file has to be created and placed in the eNOEprogram/lib folder. Importantly, this additional file has to contain the additional entries, while also keeping those in the present files.

Additional protein nomenclature convention files may also be created and placed in the eNOEprogram/lib folder if the convention is different from the standard ones offered by eNORA2.

Data structures (variables)

In order for a user to write additional matlab scripts and obtaining functionality not implemented in eNORA2 we provide with figures 10-12 an explanation of what the data structures contain.

Fig 10. Example of a sigmaData structure with fieldnames explained.

Fig 11. Example of a sigmaData(index).buildups structure with fieldnames explained.

Fig 12. Example of a sigmaData(index).decay structure with fieldnames explained.

References

[1] Recommendations for the Presentation of NMR Structures of Proteins and Nucleic Acids, J. Mol. Biol., 280, 933-952, 1998.

John L. Markley, Ad Bax, Yoji Arata, C. W. Hilbers, Robert Kaptein, Brian D. Sykes,  Peter E. Wright, and Kurt Wüthrich

[2] The Experimental Accuracy of the Uni-directional Exact NOE, J. Magn. Reson.,259, 32-46, 2015.

Dean Strotz, Julien Orts, Martina Minges, and Beat Vögeli

[3] Relaxation Matrix Analysis of Spin Diffusion for the NMR Structure Calculation with eNOEs, J. Chem. Theory Comput., 8, 3483-3492, 2012

Julien Orts, Beat Vögeli, and Roland Riek

[4] Extending the eNOE Data Set of Large Proteins by Evaluation of NOEs with Unresolved Diagonals, J. Biomol. NMR, 62, 63-69, 2015

Celestine Chi, Dean Strotz, Roland Riek, and Beat Vögeli